ESP32-S3 Heater Plate Control System — Wiring Schematic HeatsinkLab Wind Tunnel — Breadboard Reproduction Guide

Version: 1.0  |  Date: 2026-04-04

1. Component List

| Ref | Component | Description | Interface | Notes |
| --- | --- | --- | --- | --- |
| U1 | ESP32-S3-DevKitC-1 (WROOM-1-N16R8) | Main controller, 16MB Flash, 8MB PSRAM | USB + GPIO | Powered by USB; onboard 3.3V LDO |
| U2 | XY-MOS MOSFET Driver Module | PWM-controlled MOSFET switch for heater | PWM signal | VIN 5–24V (set to 12V); 3.3V logic compatible |
| U3 | INA226 Power Monitor (R100 shunt) | Measures heater current, voltage, power | I2C @ 0x40 | A0=GND, A1=GND; shunt nominal 100mOhm, calibrated 35.3mOhm |
| U4 | MAX6675 Thermocouple Module | K-type thermocouple to SPI digital | SPI (bit-bang) | 3.3V supply; CS active-low |
| PS1 | Lab Power Supply | 12V DC heater supply | Terminal | Set to 12.0V DC |
| TC1 | K/T-Type Thermocouple | Temperature probe on heater plate | Screw terminals (U4) | Polarity critical: + to T+, – to T– |
| HL1 | Heater Plate | Resistive heating element | Screw terminals (U2) | OUT+ to HL1 to OUT– |

2. Power Rails

| Rail | Source | Voltage | Consumers | Notes |
| --- | --- | --- | --- | --- |
| +12V | PS1 (Lab PSU) | 12.0V DC | XY-MOS VIN+ | Positive terminal of lab supply; high-current path |
| +3.3V | U1 onboard LDO | 3.3V | INA226 VCC, MAX6675 VCC | Use DevKit 3V3 pin only; do not add external regulator |
| GND | Common node | 0V | All modules + PSU– | ALL grounds share one node on breadboard |
| USB 5V | PC USB port | 5V | U1 only | USB-C on DevKitC-1; powers ESP32 only |

IMPORTANT: The 12V power circuit and the 3.3V logic circuit share only a single common GND node. Never connect the 12V rail to any ESP32-S3 or module logic pin.

3. GPIO Pin Assignments

| GPIO | Direction | Signal Name | Connected To | Wire Colour | Notes |
| --- | --- | --- | --- | --- | --- |
| GPIO 4 | OUTPUT | HEATER_PWM | XY-MOS TRIG/PWM | Orange | PWM ch.0; 8-bit; ledc peripheral |
| GPIO 5 | OUTPUT | FAN_PWM | (not yet connected) | Yellow | Reserved for future fan |
| GPIO 11 | INPUT | TC_SO (MISO) | MAX6675 SO | Blue | Soft SPI; read-only |
| GPIO 12 | OUTPUT | TC_CS | MAX6675 CS | Green | Active LOW |
| GPIO 13 | OUTPUT | TC_SCK | MAX6675 SCK | White | Soft SPI clock |
| GPIO 15 | I/O | I2C_SDA | INA226 SDA | Purple | 4.7kΩ pull-up to 3.3V required |
| GPIO 16 | OUTPUT | I2C_SCL | INA226 SCL | Grey | 4.7kΩ pull-up to 3.3V required |
| 3V3 pin | PWR OUT | +3.3V | INA226 VCC, MAX6675 VCC | Red | Onboard LDO; ~500mA max |
| GND pin | PWR | Common GND | All module GNDs + PSU– | Black | Use multiple GND pins if needed |

4. Module-by-Module Wiring

4.1 XY-MOS MOSFET Driver Module

| XY-MOS Pin | Connect To | Wire | Notes |
| --- | --- | --- | --- |
| VIN+ | PS1 +12V terminal directly | Red (heavy) | High-current; use 22AWG min |
| VIN– | Common GND | Black (heavy) | High-current return |
| OUT+ | INA226 IN+ | Red (heavy) | Switched 12V to heater |
| OUT– | Heater plate – terminal | Black (heavy) | Return; back to GND |
| GND | Common GND (logic) | Black | Logic ground; must also tie to common |
| TRIG/PWM | ESP32 GPIO 4 | Orange | 3.3V PWM; no level shifter needed |

Note: The XY-MOS has two ground connections: power GND (VIN–) and signal GND (GND pin). Both must connect to the common ground node.

4.2 INA226 Power Monitor Module

| INA226 Pin | Connect To | Wire | Notes |
| --- | --- | --- | --- |
| IN+ | XY-MOS OUT+ | Red | Switched 12V from MOSFET output; measures load current |
| IN– | Heater plate + | Red | Exit of shunt; goes to heater plate positive terminal |
| VBS | Same node as IN– | Short jumper | Bus voltage sense point |
| ALE | Not connected | — | Alert unused in firmware |
| SDA | ESP32 GPIO 15 | Purple | 4.7kΩ pull-up to 3.3V on breadboard |
| SCL | ESP32 GPIO 16 | Grey | 4.7kΩ pull-up to 3.3V on breadboard |
| GND | Common GND | Black | Logic ground |
| VCC | ESP32 3V3 pin | Red (thin) | 3.3V logic supply |

Note: I2C address 0x40 — A0 and A1 are GND by default on this module. Shunt marked R100 (100mΩ nominal) but firmware calibrates to 35.3mΩ — do not change without re-measuring.

4.3 MAX6675 Thermocouple Module

| MAX6675 Pin | Connect To | Wire | Notes |
| --- | --- | --- | --- |
| GND | Common GND | Black | Logic ground |
| VCC | ESP32 3V3 pin | Red | 3.3V supply |
| SCK | ESP32 GPIO 13 | White | SPI clock |
| CS | ESP32 GPIO 12 | Green | Chip select, active LOW |
| SO | ESP32 GPIO 11 | Blue | Data out to ESP32 (MISO) |
| T+ | Thermocouple + lead | Thermocouple wire | Check probe markings for polarity |
| T– | Thermocouple – lead | Thermocouple wire | Reverse = wrong readings |

Note: MAX6675 uses software SPI (bit-bang). INA226 uses I2C. No bus conflict exists between them.

5. System Block Diagram

+---------------------------------------------------------------------------+ ¦ 12V POWER CIRCUIT (High Current) ¦ ¦ ¦ ¦ +----------+ +-------------+ +-----------------+ +---------+ ¦ ¦ ¦ LAB PSU ¦ ¦ XY-MOS ¦ ¦ INA226 Module ¦ ¦ HEATER ¦ ¦ ¦ ¦ 12V DC ¦ ¦ MOSFET ¦ ¦ (R100 shunt) ¦ ¦ PLATE ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ 12V+---+--->¦VIN+ OUT+--+--->¦IN+ IN- -----+--->¦ + ¦ ¦ ¦ ¦ ¦ ¦ (PWM ctrl) ¦ ¦ (shunt here) ¦ ¦ ¦ ¦ ¦ ¦ GND----+--->¦VIN- OUT---+--->¦--- GND----------+--->¦ - ¦ ¦ ¦ +----------+ +------+------+ +-------+---------+ +---------+ ¦ ¦ TRIG¦ SDA,SCL,VCC,GND ¦ +-------------------------¦------------------------------------------------+ ¦ +-------------------------¦------------------------------------------------+ ¦ 3.3V LOGIC CIRCUIT ¦ ¦ ¦ ¦ ¦ +-----------------------+------------------------------------------+ ¦ ¦ ¦ ESP32-S3-DevKitC-1 (USB powered) ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ GPIO 4 --------------------------------> XY-MOS TRIG/PWM ¦ ¦ ¦ ¦ GPIO 15 (SDA) <------------------------------> INA226 SDA ¦ ¦ ¦ ¦ GPIO 16 (SCL) --------------------------------> INA226 SCL ¦ ¦ ¦ ¦ GPIO 13 (SCK) --------------------------------> MAX6675 SCK ¦ ¦ ¦ ¦ GPIO 12 (CS) --------------------------------> MAX6675 CS ¦ ¦ ¦ ¦ GPIO 11 (SO) <------------------------------ MAX6675 SO ¦ ¦ ¦ ¦ 3V3 pin --------------------------------> INA226 VCC ¦ ¦ ¦ ¦ 3V3 pin --------------------------------> MAX6675 VCC ¦ ¦ ¦ ¦ GND --------------------------------> All GNDs ¦ ¦ ¦ +-------------------------------------------------------------------+ ¦ +---------------------------------------------------------------------------+

6. I2C Bus Details

| Parameter | Value |
| --- | --- |
| Bus Master | ESP32-S3 (Arduino Wire library) |
| SDA Pin | GPIO 15 |
| SCL Pin | GPIO 16 |
| Pull-up Resistors | 4.7 kΩ from SDA to 3.3V AND 4.7 kΩ from SCL to 3.3V |
| Bus Speed | 100 kHz (Wire default) |
| INA226 Address | 0x40 (A0=GND, A1=GND) |
| Devices on Bus | INA226 only (no conflicts) |

7. Critical Warnings

WARNING 1: HIGH VOLTAGE / BURN RISK The heater plate, OUT+/OUT– wires, VIN+/VIN– wires, and INA226 IN+/IN– carry 12V DC at several amps. Keep all high-voltage wiring away from ESP32-S3 logic pins at all times.

WARNING 2: GROUND MUST BE COMMON The PSU negative terminal MUST share the same GND rail as the ESP32-S3. Separated grounds will prevent PWM control from working and corrupt INA226 readings.

WARNING 3: SHUNT CALIBRATION The INA226 module shunt is marked R100 (nominally 100mΩ). Firmware uses setMaxCurrentShunt(2.0f, 0.0353f), meaning the physical shunt measured ~35.3mΩ. Do not alter this value without re-measuring your specific module.

WARNING 4: THERMOCOUPLE POLARITY K-type thermocouples are polarity-sensitive. Swapping + and – will produce inverted or erratic temperature readings. Verify polarity against the probe’s datasheet or markings before powering on.

WARNING 5: MOSFET MODULE RATINGS The XY-MOS module has a maximum rated voltage and current. Do not connect heater loads that exceed these limits.

8. Breadboard Reproduction Checklist

Connect all GND nodes: PSU–, ESP32 GND, INA226 GND, MAX6675 GND, XY-MOS GND → single common rail

Connect ESP32 3V3 pin to INA226 VCC and MAX6675 VCC

Install 4.7 kΩ pull-up resistors: SDA (GPIO 15) to 3.3V and SCL (GPIO 16) to 3.3V

Wire INA226 IN+ to XY-MOS OUT+; INA226 IN– to Heater plate +

Connect INA226 VBS to the same node as IN–

Connect XY-MOS VIN– to GND; also connect XY-MOS signal GND pin to GND

Wire ESP32 GPIO 4 to XY-MOS TRIG/PWM

Wire MAX6675 SPI: SCK→GPIO 13, CS→GPIO 12, SO→GPIO 11

Wire MAX6675 VCC→3V3, GND→GND

Attach K-type thermocouple to MAX6675 T+/T– with correct polarity

Connect heater plate to XY-MOS OUT+ and OUT–

Power ESP32 via USB to PC; power heater circuit via lab PSU at 12V

Open serial monitor and verify "INA226 OK" before enabling heater output

9. Firmware Pin Constants Reference

| Constant | Value | Purpose |
| --- | --- | --- |
| heaterMosfetPin | GPIO 4 | PWM output to XY-MOS TRIG |
| fanPwmPin | GPIO 5 | Fan PWM (reserved, not wired) |
| thermocoupleSCK | GPIO 13 | MAX6675 clock |
| thermocoupleCS | GPIO 12 | MAX6675 chip select |
| thermocoupleSO | GPIO 11 | MAX6675 data (MISO) |
| i2cSdaPin | GPIO 15 | INA226 SDA |
| i2cSclPin | GPIO 16 | INA226 SCL |
| INA226 address | 0x40 | I2C address (A0=A1=GND) |
| Shunt value | 0.0353 Ohm | Calibrated (see Warning 3) |
| Max current | 2.0 A | INA226 safety limit |
| Averaging | 1024 samples | Noise rejection for PWM switching |
| PWM channel | 0 | ledc peripheral channel |